October,13,  04

Nevis Labs, Columbia University

136 So. Broadway

Irvington, NY 10533

Dear Sirs:

Enclosed are the files for our board ‘Backplane’ version 2.0 .

The finished board must meet the following requirements:

1. Board size 373 x 426.9 mm

Tolerance on the size is +-0.5mm.

2. Material FR4

3. Board thickness 2.5mm +20%-0%

4. Finish immersion gold.

5. Connector holes:

0.725+-0.075 mm plated through hole. These are the holes for press fit

connector.

6. Use tolerance of +-0.5mm for hole-to-edge dimensions.

7. Other holes: specified drill +10%-0% plated through hole

8. The netlist information (IPC-356 format) is not included. We can provide only netlist as  generated by the PowerLogic o PowerPCB programs.

If you have any questions or problems, please contact Jaroslav Ban at 914/591-2866,

FAX No.   914/591-4540.

Thanks.