







9 June 05








Nevis Labs, Columbia University









136 So. Broadway









Irvington, NY 10533

Dear Sirs:

Enclosed are the files for our board ‘GAB_V2’. 
The finished board must meet the following requirements:

1.Board size- 400.0 +-0.2   x   366.67 +-0.2 mm
The coordinate system can be referenced from the three un-plated holes  added as reference holes.  The center points of these holes are at coordinates (x,y) in millimeters
of  (22.35, 360), (22.35, 5.05) and (344,5.05) where the origin (0,0) of the 

coordinate system is located at the bottom left corner of the board outline. 
Use board dimensions as they appear in the Gerber data for the top layer photo plot.
2.Stackup                             2.2      mils 
                                   S        4.9            
                                             1.3

                                   S        5.5

                                             1.3

                                   S        4.5

                                             1.3

                                   S        5.5

                                             1.3

                                   S        5.5
                                             1.3

                                  S         5.5
                                             1.3

                                  S         5.5
                                             1.3

                                   S        5.5

                                             1.3

                                   S        4.5

                                             1.3

                                   S        5.5

                                             1.3

                                   S        4.9

                                             2.2

The layers are labeled inside the data files.
3.Board thickness- 0.076” +-10%
4.Solder Mask- two (2) sides
Our Solder Mask Photo-plots are ‘Real’ pad size. We would like you to create a 2 mil (0.002”) clearance around the pads. We would like a 50 mil clearance around the fiducial pads (50 mil diameter pads with no holes).

5.Vias are expected to be finished with immersion gold as other pads. 
6.Via Hole Sizes:


 0.010”  drilled hole size- 0.010” +0.000/-0.010” diameter after plating

                         Tolerance  for non-via holes is +/-0.003”.

7. Remaining hole sizes should be finished hole size.

8.Finish – immersion gold .

9.No controlled impedance requirements.
10.Full electrical test .

11.Finished board warpage not to exceed 0.005”/” in both diagonal directions     

     (approx.25”).

12. The net-list information (IPC-356 format) is not included. We can provide only 
      net-list as   generated by the PowerLogic o PowerPCB programs.

13. Use IPC-6012 for building and IPC-600 for inspection.
                                                                          Thanks.

