18 August 06

Nevis Labs, Columbia University

136 So. Broadway

Irvington, NY 10533

Dear Sirs:

Enclosed are the files for our ‘PCIexpress’ board version 1.0 .

The finished board must meet the following requirements:

1.Board size- 167.65 +-0.2   x   111.15 +-0.2 mm , material FR4

- Use board dimensions as they appear in the Gerber data for the top layer photo plot.

2. Proposed stack-up : top layer                        0.6 mils

prepreg    4    mils

layer 2                          1.2  mils

laminate   5    mils

layer 3                          1.2  mils

prepreg  10     mils

layer 4                           2.6  mls

laminate  10    mils

layer 5                           2.6  mils

prepreg   10    mils

layer 6                            1.2 mils

laminate    5    mils

layer 7                            1.2 mils

prepreg     4    mils

bottom layer                   0.6 mils

The layers are labeled inside the data files.

3.Board thickness- 0.062” +0 -10%

Use tolerance of +-0.005" for hole-to-edge dimensions.

4.Solder Mask- two (2) sides

Our Solder Mask Photo-plots are ‘Real’ pad size. We would like you to create a 2 mil (0.002”) clearance around the pads.

5.Vias are expected to be finished with immersion gold as other pads.

6.Via Hole Sizes:

0.010”  drilled hole size- 0.010” +0.000/-0.010” diameter after plating

- Tolerance  for non-via holes is +/-0.003”.

7. Remaining hole sizes should be finished hole size.

8.Finish – immersion gold .

9. Three dif pairs from PCIe connector  100 Ohms +-5% differential, controlled

impedance

10. Thieving allowed.

11. Edge connector - gold

12.Full electrical test .

13.Finished board warpage not to exceed 0.005”/” in both diagonal directions

(approx.25”).

14. The net-list information (IPC-356 format) is not included. We can provide only

net-list as   generated by the PowerLogic o PowerPCB programs.

15. Use IPC-6012 for building and IPC-600 for inspection.

Thanks.