







18 August 06








Nevis Labs, Columbia University









136 So. Broadway









Irvington, NY 10533

Dear Sirs:

Enclosed are the files for our ‘PCIexpress’ board version 1.0 . 
The finished board must meet the following requirements:

1.Board size- 167.65 +-0.2   x   111.15 +-0.2 mm , material FR4
Use board dimensions as they appear in the Gerber data for the top layer photo plot.
2. Proposed stack-up : top layer                        0.6 mils
                                                          prepreg    4    mils

                                     layer 2                          1.2  mils

                                                          laminate   5    mils
                                     layer 3                          1.2  mils
                                                          prepreg  10     mils
                                     layer 4                           2.6  mls
                                                          laminate  10    mils
                                     layer 5                           2.6  mils

                                                          prepreg   10    mils
                                     layer 6                            1.2 mils
                                                          laminate    5    mils 
                                     layer 7                            1.2 mils
                                                          prepreg     4    mils
                                     bottom layer                   0.6 mils

The layers are labeled inside the data files.
3.Board thickness- 0.062” +0 -10%
   Use tolerance of +-0.005" for hole-to-edge dimensions.
4.Solder Mask- two (2) sides
Our Solder Mask Photo-plots are ‘Real’ pad size. We would like you to create a 2 mil (0.002”) clearance around the pads. 
5.Vias are expected to be finished with immersion gold as other pads. 
6.Via Hole Sizes:


 0.010”  drilled hole size- 0.010” +0.000/-0.010” diameter after plating

                         Tolerance  for non-via holes is +/-0.003”.

7. Remaining hole sizes should be finished hole size.

8.Finish – immersion gold .

9. Three dif pairs from PCIe connector  100 Ohms +-5% differential, controlled    

    impedance
10. Thieving allowed.

11. Edge connector - gold

12.Full electrical test .

13.Finished board warpage not to exceed 0.005”/” in both diagonal directions     

     (approx.25”).

14. The net-list information (IPC-356 format) is not included. We can provide only 
      net-list as   generated by the PowerLogic o PowerPCB programs.

15. Use IPC-6012 for building and IPC-600 for inspection.
                                                                          Thanks.

