Stackup drawing is in the file “FEB2_stack_up_adopted for Lm_power board.xlsx”. It is for information only. The only important thing is the distance between TOP/L2 that should be the thinner: 50µm preferrably.

Please, make the stack-up symmetrical.

Board thickness is not critical.

General tolerances +/-0.05mm unless otherwise stated.

Vias-in-pads must be filled & capped. All other vias can be optionally filled & capped.

Soldemask not to be enlarged or modified by the PCB manufacturer.

LDI should be used. Soldermask should not be clipped.