Changes for FEB2 2.6 board.

(as of 12/18/2024)

green color  …done

red      color  … to be done

Common changes:

Changes in BOM  and .sch .

Change  PS connector pinout to have 2*48V lines and 8*PGND lines

Add 48V lines “termination”/filtering    1nF/10nF/100nF/1uF/100V

Add 3 extra G1 ground connections

Add power bus pins insulation with inductor (and option to remove the insulation)

Change R83 to 32kOhm           Calchip # RM04F3242CT    32.4K

Change R3_Mx to be 5k Ohm Calchip # RM04F4991CT    4.99K

Add  100k pull down resistors for all input signals.

New PS connector pinout: PGND (pin6 and pin10) +48V (pin4 and pin8).

Changes in PCB.

- Move 3 cooling holes

- ALFE_2p5_LEFT  and Alfe_2P5_RIGHT monitoring

- LSB1_VCC,  LSB2_VCC monitoring

- BPOL48V PTAT monitoring

- bPOL12V Vin_left and bPOL12V Vin_right monitoring

- Move M23_C11 M23_C33 farther from bpol48v connector alignment pin

- Move M23_C22 and M23_C46 from bpol48V modules

- Add 3 more PGND to GND connection (G1 type)

- Add 470nH inductor to PGND and +48V path

- A new Silkscreen Top and Silkscreen Bottom are generated

- During the tape generation:  increase the flooding distance to the cooper to 4.5 mils (

and check the solder mask generation)

Additional EM variant  changes:

-make sure that only one PCB type is manufactured

(strangely enough HEC variant is a baseline as it contains more pcb parts )

-make DNP pull-down all 128 100k resistors (at each input signal)

Additional HEC variant  changes:

-Populate  all 4.3M resistors (R_HPS_{1 to 32}) Yageo # RC0402FR-074M3L

-DNP all input HV protection diodes BAV99S and BAV70

-DNP all Toko inductors

-Rs=0   CALCHIP # RM04J000CT

- change coupling  Capacitor from 10u to 1u  (I put here SAMSUNG # CL05A105MQ5NNNC part)

-change all ALFE2_BGA chips to HPS chips (part name is HPS ALFE2_BGA chip )