Three changes:

- The readback counter return 7 32 bits words instead of  5 32 bits words

OP 21 counters

| bits | 15:8 | 7:5 | 4:0 |
| --- | --- | --- | --- |
|  | 0xff | 0 | Crate Number |

| bits | 15:8 | 7:5 |
| --- | --- | --- |
|  | 0x55 | 0xaa |

| bits | 15:0 |
| --- | --- |
|  | SuperNova Frame counter [15:0] |

| bits | 15:8 | 7:5 |
| --- | --- | --- |
|  | 0x0 | SuperNova Frame counter [23:16] |

| bits | 15:0 |
| --- | --- |
|  | Neutrino event counter [15:0] |

| bits | 15:8 | 7:5 |
| --- | --- | --- |
|  | 0x0 | Neutrino event counter [23:16] |

| bits | 15:0 |
| --- | --- |
|  | Neutrino event readout counter [15:0] |

| bits | 15:8 | 7:5 |
| --- | --- | --- |
|  | 0x0 | Neutrino event readout counter [23:16] |

| bits | 15:0 |
| --- | --- |
|  | SuperNova event readout counter [15:0] |

| bits | 15:8 | 7:5 |
| --- | --- | --- |
|  | 0x0 | SuperNova event readout counter [23:16] |

| bits | 15:8 | 7:0 |
| --- | --- | --- |
|  | Neutrino stream Hamming code error correct counter | Neutrino stream Hamming code error detect counter |

| bits | 31:24 | 23:16 |
| --- | --- | --- |
|  | SuperNova stream Hamming code error correct counter | SuperNova stream Hamming code error detect counter |

| bits | 15:11 | 10:0 |
| --- | --- | --- |
|  | NA | History FIFO word count |

| bits | 31:16 |
| --- | --- |
|  | 0x0 |

- History FIFO read

OP 20     read history FIFO data word

| bits | 15:0 |
| --- | --- |
|  | NA |

Return  2 32 bits data word

| bits | 15:8 | 7:5 | 4:0 |
| --- | --- | --- | --- |
|  | 0xff | 0 | Crate Number |

| bits | 31:24 | 23:16 |
| --- | --- | --- |
|  | 0x55 | 0xaa |

| bits | 31:0 |
| --- | --- |
|  | History word |

History word format (1 for neutrino, 2 for supernova)

| bits | 31 | 30 | 29 | 28 | 27 | 26-24 | 23:0 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Token 1 | Token 2 | Trig | Busy 1 rising edge | Busy 2 rising edge | NA | Frame counter |

Issue a new token, means previous event transfer is done.

Reading the history word, one should first read the read back counters. Base on the history FIFO word count N, one should read the history FIFO data word N times. The FIFO will get refilled again when history tags happened. So the read process loop back to the history FIFO word count again.

- When readout the trigger events, the supernova readout counter will advance by 3. This is to cover to 4 frame readout of the trigger events.